The internet - Answer key

Y12 - Cambridge International 9618  |  Bit streaming and 2.2  |  Teacher copy - not for students

ANSWER KEY - TEACHER COPY   Answers and marking points for every task on the worksheet and student booklet (same task numbers).

"Official" answers come from the published Cambridge mark schemes for the paper cited. "Suggested" answers are written in mark-scheme style from the textbook (Hodder 9618, sections 2.1.8 and 2.2) and the official points for similar questions. There are no 9618 Paper 1 questions (2021-2025) on URLs or how DNS works, so Task 5(c)-(d) are exam-style. Total: 71 marks.

Task 1 Recall test - network hardware [12]

Re-teach slides 3-4 if most of the class scores under 7/12 or confuses MAC and IP addresses.

Suggested - Q1 [6], 1 mark per row

| Purpose | Device |
| --- | --- |
| Forwards data only to the intended device, using MAC addresses | Switch |
| Forwards packets between networks using IP addresses and a routing table | Router |
| Connects wireless devices to a wired network using radio waves | Wireless access point (WAP) |
| Restores a weakened signal so it can travel further | Repeater |
| Connects two LAN segments that use the same protocol | Bridge |
| Converts digital data to analogue for a telephone line, and back | Modem |

Suggested - Q2 [3], 1 mark per row

MAC; IP; MAC.

Q3 Official - 9618/12 May/June 2025 Q6(e), max 3

| Mark | Marking point |
| --- | --- |
| 1 | To store the MAC addresses of devices connected to it |
| 1 | To receive packets from devices |
| 1 | To forward packets directly to the intended recipient |
| 1 | To provide a central point of connection |
| 1 | To enable connected devices to communicate |

Task 2 Bit streaming [12]

(a) Official - 9618/11 May/June 2024 Q2(e)(i) [1]

| Mark | Marking point |
| --- | --- |
| 1 | Continuous ordered flow of bits over a communication path |

(b) Official - 9618/11 May/June 2024 Q2(e)(ii), 1 mark each, max 2

| Mark | Marking point |
| --- | --- |
| 1 | Real-time is direct from the source, whereas on-demand is pre-recorded / downloaded to view later |
| 1 | Real-time cannot be re-watched, whereas on-demand can be paused, re-watched etc. |
| 1 | Real-time plays continually, whereas on-demand downloads sections / blocks and cannot play until the next section is downloaded |

(c) Official - 9618/13 Oct/Nov 2024 Q9(c), max 4

| Mark | Marking point |
| --- | --- |
| 1 | Data is compressed before transmitting |
| 1 | ... transmitted continuously as a series of bits |
| 1 | ... uploaded to a media server |
| 1 | The media server sends the data to a buffer |
| 1 | The buffer stores data from the server until the receiving device can process it |
| 1 | The receiving device receives the bit stream from the buffer |

Suggested - (d)(i) [2]

45 minutes = 2700 s (1).   4 Mbps x 2700 s = 10 800 Mb;  10 800 / 8 = 1350 MB (1).

Accept 1.35 GB. Allow follow-through for a correct /8 conversion from a wrong total.

Suggested - (d)(ii) [3]

- The broadband speed (2.5 Mbps) is lower than the bit rate of the video (4 Mbps) (1).

- So data is played from the buffer faster than it arrives; the buffer empties and the video pauses / buffers / stutters (1).

- One way to fix it (1): stream at a lower quality / resolution (lower bit rate, at or below 2.5 Mbps); get faster broadband; let the buffer fill (or download the file) before watching.

Task 3 The internet and its hardware [9]

(a) Official - 9618/11 and /13 May/June 2021 Q4(d) [3]

| Mark | Marking point |
| --- | --- |
| 1 | Both |
| 1 | Using the internet because the data is being sent on the infrastructure (the network of networks) |
| 1 | Using the WWW because she is accessing a website (stored on the webmail provider's web server) that is part of the WWW |

Examiner report: almost everyone said "both", but many could not justify it. "Because her email account is accessed through a website" just repeats the question.

(b) Official - 9618/11 May/June 2025 Q2(c), 1 mark each

| Mark | Marking point |
| --- | --- |
| 1 | Modem: converts digital data to analogue for transmission down phone lines // converts analogue to digital after transmission |
| 1 | Dedicated line: provides a direct / private connection, giving faster transmission |

(c) Official - 9618/12 May/June 2024 Q3(c)(ii), max 2

| Mark | Marking point |
| --- | --- |
| 1 | The PSTN consists of many different types of communication lines ... |
| 1 | ... therefore the digital data may need to be converted into a different form / analogue signal |
| 1 | Data is transmitted in both directions at the same time // duplex data transmission |
| 1 | Using a PSTN the communication passes through different switching centres / ISPs |

(d) Official - 9618/12 May/June 2025 Q6(b)

| Mark | Marking point |
| --- | --- |
| 1 | Land is split into cells designed for maximum line of sight |
| 1 | Each cell has a tower with an antenna which receives and transmits data |
| 1 | Data is transmitted between the tower and the phone |
| 1 | Data transmission is wireless, using low-power radio signals / frequencies |
| 1 | Multiple devices can communicate simultaneously with the same tower |

The question paper shows [2] for this part, but the published mark scheme says "1 mark each to max 4". The worksheet uses [2] - check the printed paper.

Task 4 IP addresses [15]

Official - (a) 9618/11 May/June 2023 Q4(d)(i) [5], 1 mark per gap

4;  8;  8;  16;  double colon ( :: )

Examiner report: a single colon instead of a double colon was a common wrong final answer.

(b) Official - 9618/12 Oct/Nov 2023 Q7(a), max 2

| Mark | Marking point |
| --- | --- |
| 1 | It only has four groups of digits // IPv6 has eight groups |
| 1 | It uses dotted notation instead of colons |
| 1 | It is a 32-bit / 4-byte address // IPv6 is 128 bits / 16 bytes |

Suggested - (c) [4], 1 mark per row

| Address | Answer |
| --- | --- |
| 172.16.254.1 | Valid IPv4 |
| 10.0.300.12 | Invalid - 300 is more than 255 |
| 2001:DB8:0:0:8D3:0:0:2 | Valid IPv6 - eight groups; leading zeros may be dropped |
| FE80::1::A2 | Invalid - :: used twice, so it is ambiguous |

Official - (d) 9618/12 Oct/Nov 2023 Q7(d) [4], 1 mark per box

| Type of IP address | Description |
| --- | --- |
| Public | Assigned to a device to allow direct access on the internet |
| Private | Used for internal LAN communication only |
| Static | Fixed // does not change each time the device re-joins a LAN / network |
| Dynamic | May be refreshed / changed each time the device re-joins a LAN / network |

Examiner report (Nov 2023): "an IP address that randomly changes" is not enough for dynamic.

Task 5 Subnetting, URLs and DNS [13]

Official - (a) 9618/13 Oct/Nov 2022 Q7(c)(ii) [2]

Network ID of device 1: 10.    Host ID of device 2: 4.

255.0.0.0 means only the first group is the network ID; 255.255.255.0 means the last group is the host ID. Examiner report: many candidates need to improve their understanding of subnet masks.

(b) Official - 9618/11 May/June 2023 Q4(d)(ii) [4]: 1 mark for each benefit, 1 for its expansion

| Mark | Marking point |
| --- | --- |
| 2 | Reduces traffic / improves speed ... because data stays in its own subnet |
| 2 | Improves security ... so that not all devices can access all areas of the network |
| 2 | Easier maintenance ... because only one subnetwork may need to be taken down |

Examiner report: many answers gave the general benefits of any network rather than of subnetting. Other accepted benefits elsewhere: easier expansion; hides the complexity of the network.

Suggested - (c) [3], 1 mark each

| Part | Answer |
| --- | --- |
| Protocol | https |
| Domain name | www.cambridgeinternational.org |
| Path | /programmes-and-qualifications/ |

Suggested - (d) [4], 1 mark per point, max 4

- The browser sends the domain name (from the URL) to a DNS server.

- The DNS server searches its database / cache for the matching IP address.

- If it is not found, the request is passed on to another (higher-level) DNS server.

- The IP address is returned to the browser (and cached by the DNS server).

- The browser sends a request for the web page to the web server at that IP address.

- The web server sends the page back and the browser displays it.

Task 6 Extension [10]

(a) Official - 9618/11 May/June 2025 Q2(b)(i), max 4

| Mark | Marking point |
| --- | --- |
| 1 | Video is data-intensive / large, so compression reduces the file size ... |
| 1 | ... so less bandwidth is used |
| 1 | ... so there is less buffering / delay |
| 1 | So people are not left behind in the conversation (it stays in real time) |
| 1 | So users with lower bandwidth can still take part |

(b) Official - 9618/11 May/June 2025 Q2(b)(ii), max 3 (no mark for naming lossy on its own)

| Mark | Marking point |
| --- | --- |
| 1 | Lossy reduces the file size more than lossless ... |
| 1 | ... so less bandwidth is needed and there is less buffering |
| 1 | It removes data that cannot be perceived // e.g. lowers the resolution / sample rate, which is not noticed in a video call |

Examiner report (Nov 2023, similar question): say why the method is chosen, not how it works.

Suggested - (c) [3]

- 200 = 11001000; 224 = 11100000 (1).

- 11001000 AND 11100000 = 11000000 = 192 (1).

- The device is in the subnet 172.16.5.192 (1).   (Its host ID is 01000 = 8.)

Exit question (slide 26) - model answer

(a) [2] Real-time: a live event streamed as it happens and cannot be paused or rewound (1); on-demand: stored files streamed when the user chooses, and can be paused / rewound (1).

(b) [2] The browser sends the domain name www.bbc.co.uk to a DNS server (1); the DNS server looks up the matching IP address and returns it to the browser, which then contacts the web server at that address (1).

(c) [1] Any valid IPv6 address with :: used once, e.g. 2001:DB8::1.