Network hardware - Answer key

Y12 - Cambridge International 9618  |  2.1.6  |  Teacher copy - not for students

ANSWER KEY - TEACHER COPY   Answers and marking points for every task on the worksheet and student booklet (same task numbers).

"Official" answers come from the published Cambridge mark schemes for the paper cited. "Suggested" answers are written in mark-scheme style from the textbook (Hodder 9618, section 2.1.6) and the official points for similar questions. Total: 61 marks.

Task 1 Recall test [10]

Re-teach if most of the class scores under 6/10, or under 2/4 on Q2.

Suggested - Q1 [3]

| Statement | Answer |
| --- | --- |
| Transmits data as pulses of light | Fibre optic |
| The cheapest cable and the most common in LANs, but affected most by interference | Twisted pair |
| Commonly used in MANs and by cable television companies | Coaxial |

Official - 9618/13 Oct/Nov 2024 Q9(b) [4]: 1 mark for each medium, 1 mark for its description

| Transmission medium | Description |
| --- | --- |
| Fibre-optic cable | Transmits data as pulses of light |
| Radio waves / microwaves | Transmit (electromagnetic) waves on different frequencies |

Also credit satellite for the medium if it is described correctly (e.g. microwaves relayed by a satellite).

Suggested - Q3 [3] (9618/11 May/June 2021 Q4(c)(ii)) - credit either choice if it is justified; summary of the official points

- Wired: faster / higher bandwidth, so films stream without buffering; more reliable / less interference, so games do not lag or drop out; more secure.

- Wireless: freedom of movement - she can watch anywhere in range; no (or fewer) cables needed; easy to connect mobile devices.

- Typically 1 mark for the tick (only if justified) and up to 2 for the justification; a justification that does not match the tick earns nothing.

Task 2 Identify the hardware [12]

Suggested - (a) [8] - 1 mark per device

| Label | Device | Label | Device |
| --- | --- | --- | --- |
| A | Switch | E | NIC (network interface card) |
| B | Server (file server) | F | Bridge |
| C | Wireless access point (WAP) | G | Repeater |
| D | WNIC (wireless network interface card) | H | Router |

Accept "router" for A only if the student explains it is acting as the central device - but the router is H here, so the best answer is switch. Do not accept "hub" for F or "modem" for H.

Official - 9618/12 May/June 2022 Q9(a) [4] - 1 mark for each completed name or description

| Device | Description |
| --- | --- |
| Router (see note) | Receives and sends data between two networks operating on the same protocol |
| Wireless Network Interface Card (WNIC) | Hardware component that allows a device to connect to a wireless network // provides a MAC address to the device to identify it on the wireless network |
| Repeater | Restores the digital signal so it can be transmitted over greater distances |
| Wireless Access Point (WAP) | Hardware component that provides radio communication from the central device to nodes on the network (and vice versa) |

Note on row 1. The published mark scheme gives "Router" and lists no alternative. The description ("between two networks operating on the same protocol") matches the textbook definition of a bridge almost word for word, and 9618/11 Oct/Nov 2023 Q2(b) credits exactly this wording for a bridge. Accept bridge in class and use the row to discuss the difference: a router can join networks that use different protocols.

Task 3 Apply [10]

Official - (a) 9618/11 May/June 2021 Q4(c)(i) [2] - 1 mark for rows 1 and 2 correct, 1 mark for rows 3 and 4 correct

| Task | Answer |
| --- | --- |
| Receives packets from devices | Performed by router |
| Finds the IP address of a Uniform Resource Locator (URL) | Not performed - a DNS server does this |
| Directs each packet to all devices attached to it | Not performed - this is what a hub does |
| Stores the IP and/or MAC address of all devices attached to it | Performed by router |

The tick positions are not visible in the text version of the mark scheme; the answers above follow the official router marking points (receives packets; maintains a table of MAC and IP addresses). Check against the printed scheme if needed.

Suggested - (b) [4]

| Central device | PC2 | PC3 | PC4 |
| --- | --- | --- | --- |
| Hub | Yes | Yes | Yes |
| Switch | No | Yes | No |

1 mark for each row correct. Two reasons, 1 mark each: the switch sends data only to the intended device, so other devices cannot read it (more secure); less unnecessary traffic, so bandwidth is not wasted / the network is faster; fewer collisions.

Suggested - (c) [4] - 1 mark per row

| Statement | Answer |
| --- | --- |
| It is set by the manufacturer of the NIC or WNIC | MAC address |
| A router reads it to forward a packet to another network | IP address |
| It can change when a laptop joins a different network | IP address |
| A switch reads it to decide which port to send data out of | MAC address |

Task 4 Exam questions - the router [7]

(a) Official - 9618/11 Oct/Nov 2023 Q2(a), max 3

| Mark | Marking point |
| --- | --- |
| 1 | Receives packets from the internet / external network |
| 1 | Implements a firewall |
| 1 | Analyses the destination IP address of each packet |
| 1 | Forwards the packet towards its destination // sends packets onto the local or external network ... |
| 1 | ... using the routing table |
| 1 | Maintains / updates the routing table |
| 1 | Allocates private IP addresses |
| 1 | Finds the most efficient route to the destination |
| 1 | Changes the packet format for transmission over the next network |
| 1 | NAT - lets several devices share one public IP address |

Examiner report: many answers were general knowledge; "it connects the LAN to the internet" is already in the question and earns nothing.

(b) Official - 9618/12 May/June 2024 Q3(c)(i), max 2

| Mark | Marking point |
| --- | --- |
| 1 | Receives packets from the internet |
| 1 | Analyses the destination IP address of each packet |
| 1 | Forwards the packet towards its destination ... |
| 1 | ... using the routing table |
| 1 | Maintains / updates the routing table |
| 1 | Finds the most efficient route to the destination |

(c) Official - 9618/12 Oct/Nov 2023 Q7(b)(ii), max 2

| Mark | Marking point |
| --- | --- |
| 1 | The router determines the recipient's destination address ... |
| 1 | ... by using a routing table |
| 1 | The router transmits the data directly / only to the recipient laptop |

Task 5 Exam questions - LAN devices [12]

(a) Official - 9618/11 Oct/Nov 2023 Q2(b), 1 mark per device

| Mark | Marking point |
| --- | --- |
| Switch | To allow two or more devices to communicate with one another // to connect individual devices to each other // to receive transmissions and forward them to their destination |
| WAP | To allow connection of devices (to the central device) using radio signals / Wi-Fi // to allow the central device to send / receive radio (Wi-Fi) signals // to allow wireless-enabled devices to connect to a wired network |
| Bridge | To connect two LANs / segments with the same protocol // to transmit data between two networks with the same protocol |

Examiner report: "allows a device to connect wirelessly" for a WAP was not clear enough - candidates need technical knowledge of each device.

(b) Official - 9618/12 May/June 2025 Q6(e), max 3

| Mark | Marking point |
| --- | --- |
| 1 | To store the MAC addresses of devices connected to it |
| 1 | To receive packets from devices |
| 1 | To forward packets directly to the intended recipient |
| 1 | To provide a central point of connection |
| 1 | To enable connected devices to communicate |

Examiner report: a common wrong answer was "to turn the network on and off"; answers must make clear the switch sends only to the intended recipient.

(c) Official - 9618/11 Oct/Nov 2021 Q8(b), 1 mark each, max 2

| Mark | Marking point |
| --- | --- |
| 1 | Router |
| 1 | Switch |
| 1 | Hub |

Do not credit "cables", "server" or "NIC" - the question asks for devices that connect the computers to the rest of the network.

(d) Official - 9618/11 Oct/Nov 2021 Q8(c), max 4

| Mark | Marking point |
| --- | --- |
| 1 | Provides an interface to the wireless network ... |
| 1 | ... as an antenna |
| 1 | Receives analogue radio waves ... |
| 1 | ... and converts them to digital / binary |
| 1 | Checks incoming transmissions for the correct MAC / IP address ... |
| 1 | ... and ignores transmissions not intended for it |
| 1 | Encrypts / encodes the data |
| 1 | Decrypts / decodes the data |
| 1 | Takes digital input and converts it to analogue waves ... |
| 1 | ... and sends the radio waves via the antenna |

Examiner report: a frequent misconception was that the WNIC provides an IP address rather than a unique MAC address; "a WNIC functions like a NIC, but wirelessly" was not precise enough.

Task 6 Extension [10]

(a) Official - 9618/12 May/June 2025 Q6(d) [4]

| Mark | Marking point |
| --- | --- |
| 1 | Server connected directly to the switch |
| 1 | Four labelled computers, all connected directly to the switch |
| 1 | Two printers, each connected to the switch / the server / a computer |
| 1 | Router / modem connected to the switch / the server |

Examiner report: marks were lost for unlabelled or missing devices, and many candidates did not identify the device providing internet access (a router).

Suggested - (b) [6] - 1 mark per hardware item correctly justified for this library, max 6

- Fibre-optic cable for the 400 m link to the main building - too far for twisted pair (about 100 m), and fibre has low signal loss and high bandwidth. (Alternatively twisted pair with repeaters to restore the signal - credit if the 100 m limit is explained.)

- A switch in the library as the central device for the 20 desktops and printer, forwarding data only to the intended device.

- A bridge (or a switch link) joining the library segment to the school LAN, because both use the same protocol and share the same file server - a second router is not needed.

- Wireless access points (several, for 60 laptops across the library) cabled to the switch, so laptops with WNICs can join the wired network by radio.

- NICs in the desktops (usually built in) and WNICs in the laptops - each provides a MAC address.

- Twisted pair (Cat 6) cable from each desktop / printer / WAP to the switch.

- No new router or modem: internet traffic goes through the main building's router, which assigns private IP addresses and acts as the firewall.

Exit question (slide 23) - model answer

(a) [2] A switch reads the destination MAC address (1) and sends the data only to the intended computer, whereas a hub sends it to all 30 (1) - so it is more secure / wastes less bandwidth.

(b) [2] A bridge connects two LANs / segments that use the same protocol so they act as one network (1); a router forwards packets between different networks (e.g. the LAN and the internet) using IP addresses and a routing table (1).