Wired & wireless networking - Answer key

Y12 - Cambridge International 9618  |  2.1.5  |  Teacher copy - not for students

ANSWER KEY - TEACHER COPY   Answers and marking points for every task on the worksheet and student booklet (same task numbers).

Task 1 Q3 uses the official marking points from the published 9618/13 Oct/Nov 2023 mark scheme. All other answers are suggested, based on the textbook (Hodder 9618, section 2.1.5), in Cambridge mark-scheme style.

Task 1 Recall test - answers [8]

1. [3] Star; bus; mesh.

2. [1] Official (9618/13 June 2024 Q5(a)(i)): dedicated / bespoke services or storage on a remote server that is only available to the company.

3. [4] Official (9618/13 Oct/Nov 2023 Q3(a)(ii)), max 2 marks per disadvantage (point + expansion):

| Disadvantage (1 mark) | ... expansion (1 mark) |
| --- | --- |
| Possible loss of control, unlike the LAN | because the data is stored on someone else's / remote infrastructure // relies on an external agency for tasks such as backups and security |
| Requires a reliable internet connection | to access the remote data - access is more reliable with a LAN |
| Increased recurring costs | cloud provider charges must keep being paid, whereas the LAN costs are mostly one-off |

Task 2 - answers [6]

(a) [3] Fibre optic; twisted pair; coaxial.

(b) [3] Bluetooth; Wi-Fi; Bluetooth.

Task 3 - answers [10] (suggested)

(a) [2] 1 mark for a reason, 1 for the expansion, e.g.: fibre has much less signal loss / attenuation over long distances (1) so the signal can travel 2 km without needing to be boosted (1); fibre has a much higher data transfer rate (1) so many users in both buildings can share the link (1); fibre is not affected by electrical / external interference because it uses light (1).

(b) [3] Advantages, any two: easy to expand - no cables to install; customers' devices are mobile within range of the WAP; phones and tablets can connect without a cable. Disadvantage, any one: less secure as signals can be intercepted (so encryption is needed); more interference / signal can drop out; slower data transfer than wired.

(c) [2] Microwave / radio signals travel in straight lines, so the curvature of the Earth blocks the signal between distant antennae (1). The signal is beamed up to a satellite, which boosts it and sends it back down to the receiving antenna (1).

(d) [3] Any three: Bluetooth uses 79 channels / frequencies (around 2.45 GHz); a device picks a channel at random; if the channel is already in use, it picks another; communicating pairs change / hop channels several times a second; so different pairs are unlikely to use the same channel at the same time, minimising interference.

Task 4 - marking guidance [6] (suggested)

1 mark per valid point with expansion applied to the 20-floor building, max 5; 1 mark for a justified conclusion.

- For wired: faster and more stable data transfer, so staff on every floor get reliable access to servers; more secure because signals cannot be intercepted from outside the building; tends to be cheaper overall; no dead spots between floors.

- Against wired: cabling 20 floors is disruptive and needs ducts or risers; devices are not mobile; trip hazards and cables pulled out during cleaning.

- For wireless: easy to expand and reorganise floors; staff and visitors can move around with laptops, phones and tablets.

- Against wireless: signals are weakened by walls and floors, so many WAPs (and possibly repeaters) are needed on each floor; less secure - needs encryption such as WPA2; more interference; slower than wired.

- Conclusion (example): a wired backbone (e.g. fibre between floors and twisted pair to desks) with Wi-Fi access points on each floor for mobile devices - fast, reliable core links plus mobility. Credit any conclusion that is justified.

Exit question (slide 12) - model answer

Fibre-optic cable (1). Any one reason: the least signal loss over 2 km // the highest data transfer rate // not affected by electrical interference (1).